# هندسة الأسس العميقة – أسئلة الامتحان التقويمي (الدور الأول) 2026

المرحلة الرابعة – تاريخ الامتحان: 2/6/2026

1- Pile foundation could be used when there is …… located at shallow depths.

a- soft clay

b- Medium sand

c- Stiff clay

d- Loess soil

e- Expansive soil

2- The skin friction is fully mobilized when the pile settlement is equal to

a- 15-20% of pile diameter

b- 10-15% of pile diameter

c- 10-25% of pile diameter

d- 10-25 mm

e- 10-15 mm

3- The value of lambda (λ) in the determination of skin friction of piles depends on

a- Pile diameter

b- Pile embedded length

c- Undrained cohesion of the soil

d- Angle of internal friction of the soil

e- Method of construction of pile

4- The pile cap would help with transformation of the load of the structure to the soil if the cap is in contact with the ground

a- True

b- False

c- Statement only valid for clay

d- Statement only valid for sand

e- Statement only valid for cast in-situ piles

5- Piles can be divided into the following categories with the general descriptions for conventional

a- steel, concrete

b- timber, steel

c- concrete and composite piles

d- Both a and b

e- steel, concrete, timber, and composite piles

6- When necessary, steel piles are spliced by

a- welding or by riveting

b- Welding

c- Riveting

d- All above

e- Other than that

7- Piles are structural members made of

a- steel material

b- Timber material

c- concrete material

d- composites material

e- All of the mentioned

8- An advantage of preferring bored piles is

a- The concrete is not liable to damage

b- Vibration caused by driving can be avoided

c- They need no storage space

d- No requirement of special handling equipment

e- avoiding noise

9- The limiting value of qL = 0.5 pa Nq* tanφ', the effective angle is for:

a- soil close to the surface

b- friction between the pile and the soil

c- soil friction angle of bearing stratum

d- average of the friction and bearing stratum

e- all of above

10- In general, the settlement of pile group in saturated clay layer is ……… sandy layer

a- equal

b- more than

c- less than

d- less or equal

e- None of the above

11- During the casting of the precast piles, the cables are pretensioned to about

a- 950 – 1450 MN/m²

b- 800 – 1300 MN/m²

c- 900 – 1300 MN/m²

d- 800 – 1400 MN/m²

e- 950 –1350 MN/m²

12- The piles or piers supporting the tower legs must be designed for

a- compressive forces

b- tension forces

c- compressive and tension forces

d- shear forces

e- none of the mentioned

13- The minimum tip diameter for Class B timber piles should not be less than

a- 150 mm

b- 250 mm

c- 305 mm

d- 330 mm

e- 356 mm

14- What is negative skin friction?

a- A downward drag force exerted on a pile by the soil

b- A vertical force exerted on a pile by the soil

c- An upward force exerted on a pile by the soil

d- A lateral force exerted on a pile by the soil

e- None of the above

15- The maximum load which can be carried by a pile is defined as its

a- Ultimate load carrying capacity

b- Ultimate bearing resistance

c- Ultimate bearing capacity

d- Ultimate pile capacity

e- All of the mentioned

16- What minimum butt diameter is specified for Class C timber piles used permanently under the water table?

a- 150 mm

b- 330 mm

c- 200 mm

d- 356 mm

e- 305 mm

17- Which type of composite piles are increasingly used for waterfront structures?

a- Simple cast-in-place concrete piles

b- Timber-and-concrete composite piles

c- Metal strap composite piles

d- Steel-and-timber composite piles

e- Fiber reinforced polymer composite piles

18- Which type of piles are referred to as no displacement piles?

a- H-piles and open-ended pipe piles

b- Piles advanced by partial augering

c- Steel sheet piles

d- Prestressed concrete piles

e- Cast-in-place (or bored) piles

19- Underreamed piles are piles with enlargement bottom to increase the pile resistance against ……

a- Corrosive

b- Tilting

c- Lateral force

d- Uplift

e- Sinking

20- Why are steel H-section piles preferred over I-section beams?

a- H-section piles are more visually appealing

b- I-section beams have limited sizes

c- H-section piles resist corrosion better

d- I-section beams are more expensive

e- H-section piles have equal web and flange thicknesses

21- A concrete bored pile of (750mm) diameter can reach its maximum point resistance when its tip settlement reach ……

a- 75 mm

b- 187.5 mm

c- 262.5 mm

d- 300 mm

e- 10 mm

22- The technique of the pile load test in which the load is applied in increments equal to 25% of the design load until reaching the maximum test load, is called ……

a- Load-controlled test

b- Constant rate of penetration test

c- Repeated loading test

d- Cyclic loading test

e- Constant-movement increment test

23- The downward drag force exerted on a pile by the soil surrounding it until reaching its maximum point resistance is called ……

a- Skin friction

b- Negative skin friction

c- Frictional force

d- All of the above

e- None of the mentioned

24- The negative skin friction on a pile develops when

a- The soil in which it is driven is sandy soil

b- The soil surrounding the pile settles more than the pile

c- The ground water table rises

d- The soil near the tip is clay

e- All of the above

25- What is the best hammer type for precast piles to be driven near a hospital?

a- Impact hammer

b- Vibratory hammer

c- Dropped hammer

d- Hand operated hammer

e- All of the above

26- The downward drag force acting on a pile due to the movement of the surrounding is called ……

a- Skin friction

b- Negative skin friction

c- Frictional force

d- End bearing

e- None of the mentioned

27- Negative skin friction on piles

a- Is caused due to relative settlement of the soil

b- Is caused in soft clays

c- Decreases the pile capacity

d- Is caused due to lowering of water table

e- All of the above

28- The pile load test should be performed on

a- Working pile

b- Test pile

c- a and b

d- Reaction pile

e- All of the mentioned

29- In standard penetration test (SPT) for soil, the SPT number is:

a- Penetration depth of sampler in the soil for any given number of blows

b- Number of blows required for 30 cm of penetration of sampler in the soil

c- Number of blows required for 20 cm of penetration of sampler in the soil

d- Number of blows required for 10 cm of penetration of sampler in the soil

e- None of the above

30- What parameter plays a pivotal role in determining the efficiency of a pile group?

a- Length of the piles.

b- Depth of the groundwater table.

c- Number of piles in the group.

d- Soil cohesion.

e- Load-carrying capacity of individual piles.

31- Usually for a group piles, piles caps are provided for which of the following reasons

a- Stop any infiltration of water from top

b- Spread loads to all piles

c- Arrest lateral displacement

d- Increase the load bearing capacity of each pile

e- decrease the load bearing capacity of each pile

32- The factor of safety of uplift pressure

a- 1.5-2

b- ≥ 3

c- unified

d- more than 1

e- The more the better

33- What factor determines the efficiency of a pile group?

a- Soil type

b- Pile diameter

c- Pile spacing

d- Interaction between adjacent piles

e- Depth of embedment

34- How do the piles within a group act according to the simplified analysis depending on their spacing within the group?

a- Independently and randomly

b- Either as a block or as individual piles

c- Always as individual piles

d- Only as a block

e- According to their depth in the soil

35- What is the primary factor influencing elastic settlement of piles and pile groups?

a- Soil type

b- Pile material

c- Pile diameter

d- Load magnitude

e- Pile spacing

36- Which of the following factors does NOT have a direct effect on the elastic settlement of a pile?

a- Diameter of the pile

b- Cohesion of the surrounding soil

c- Modulus of elasticity of the pile material

d- Length of the pile

e- Magnitude of the applied axial load

37- Tension piles may be used beneath buildings to resist uplift from

a- hydrostatic pressure

b- structures over expansive soils

c- power transmission towers

d- all the above

e- no one of the above

38- A pile which obtains most of its load carrying capacity at the base of pile is known as

a- end bearing pile

b- friction pile

c- compaction pile

d- composite pile

e- None of these

39- Piles must be driven to a certain depth where the expansive soil are present to

a- Resist swelling effect

b- To increase pile capacity

c- To prevent soil collapse

d- All the above points

e- None of the above points

40- When piles is driven into a thick deposit of soil the load applied to the pile head is transferred to the soil through

a- Pile tip

b- Pile shaft

c- Pile tip and shaft

d- pile interaction

e- None of the above points

41- The minimum center-to-center pile spacing, d in the group piles is:

a- 1.5 D

b- 2.5 D

c- 3.5 D

d- 4.5 D

e- 5.5 D

42- In which conditions do steel piles require additional thickness to offset corrosion effects?

a- In sandy soils

b- In clay soils

c- In silty soils

d- In dry climates

e- In swamps and peats

43- If the entire load is transferred to the soil through the pile tip, it is considered to be:

a- Point-bearing piles

b- Friction piles

c- Compaction piles

d- Batter piles

e- All of the above

44- When there is no bedrock or stiff stratum at relatively short depth, the pile is called:

a- Point bearing pile

b- Friction pile

c- Plugged pile

d- Composite pile

e- Compaction pile

45- Soils with a pH greater than ……… are not as corrosive to steel piles.

a- 4

b- 5

c- 6

d- 7

e- 8

46- The vibration caused during pile driving helps densify the soil around the pile. The zone of sand densification may be as much as:

a- 1.5 times the pile diameter, in the sand surrounding the pile.

b- 4.0 times the pile diameter, in the sand surrounding the pile.

c- 3.0 times the pile diameter, in the sand surrounding the pile.

d- 2.5 times the pile diameter, in the sand surrounding the pile.

e- 2.0 times the pile diameter, in the sand surrounding the pile.

47- The maximum point resistance of the driven pile (30 cm x 30 cm) will not be mobilized until the tip of the pile has moved about ……………….

a- 3 mm to 9 mm

b- 3 cm

c- 1 mm to 3 mm

d- 2 cm

e- less than 3 mm

48- Which type of pile load test applies a continuously increasing load to maintain a constant rate of penetration?

a- Load-controlled test

b- Cyclic loading test

c- Ultimate load test

d- Constant-rate-of-penetration test

e- Constant load test

49- What is the effect of negative skin friction on piles?

a- Increases load-bearing capacity

b- Reduces effective pile capacity

c- Eliminates the need for reinforcement

d- Increases lateral stability

e- Increase end bearing capacity

50- To estimate the critical depth conservatively, it would be

a- greater than 15D

b- equal 15D

c- less than 15D

d- all of the mentioned

e- none of the mentioned

51- In clayey soils, the resistance to applied load is caused by:

a- skin friction and cohesion

b- skin friction only

c- cohesion only

d- skin friction and adhesion

e- adhesion only

52- If a group efficiency η = 0.75 and the sum of individual pile capacities is 4000 kN, the group ultimate capacity is:

a- 2500 kN

b- 3000 kN

c- 3200 kN

d- 4000 kN

e- 5333 kN

53- In a friction pile installed in uniform sand, increasing pile diameter while keeping length constant will primarily increase:

a- Point resistance only

b- Shaft resistance only

c- Both shaft and point resistance significantly

d- Neither shaft nor point resistance

e- Negative skin friction

54- In uplift (tension) piles, resistance is mainly provided by:

a- Point bearing only

b- Shaft resistance and pile self-weight

c- Passive earth pressure at tip

d- Water pressure

e- Group efficiency

55- Factory-applied epoxy coatings are mainly used on steel piles to:

a- Increase load capacity

b- Reduce corrosion of steel

c- Decrease pile length

d- Reduce installation time

e- Improve appearance