Foundation Engineering 2 - Exam Questions

Exam Date: 25/4/2024  |  Time: 90 minutes  |  Stage: Fourth

Note: Correct answers are highlighted in green.

1. Which class of timber piles is carry heavy loads:

a) Any class

b) Class C

c) Class A

d) Class B

e) Class D

2. For driven piles in sand, the vibration caused during pile driving helps densify the soil around the pile. The zone of sand densification may be as much as:

a) 1.5 times the pile diameter

b) 2.5 times the pile diameter

c) 3.5 times the pile diameter

d) 4.5 times the pile diameter

e) More than 4.5 times the pile diameter

3. Belled or under-reamed piles are:

a) concrete piles with enlarged base to resist uplift loads

b) Steel piles with plugged end

c) Helical piles resisting lateral loads

d) Piles with uniform cross section along the pile length

e) No one of above

4. Why are bridge abutments and piers usually constructed over pile foundations?

a) To achieve proper compaction of soil

b) To resist lateral loads

c) To avoid loss of bearing capacity due to soil erosion

d) To prevent damage from expansive soil

e) Preventing uplift

5. The maximum frictional resistance along the pile shaft will be fully mobilized when the relative displacement between the soil and the pile is about:

a) 5 to 10 mm

b) 2 to 3 mm

c) 10 to 15 mm

d) 7 to 10 mm

e) 4-6 mm

6. The main function of retaining walls is:

a) Stabilized of roads

b) Control erosion

c) to reduce the grades of roads

d) lateral support for earth structures

e) All of the mentioned

7. A 3 m high retaining wall is supporting a saturated sand (saturated due to capillary action) of bulk density 18 kN/m³ and angle of shearing resistance 30°. The change in magnitude of active earth pressure at the base due to rise in ground water table from the base of the footing to the ground surface shall (γw = 10 kN/m³):

a) Increase by 20 kN/m²

b) Decrease by 20 kN/m²

c) Increase by 30 kN/m²

d) Decrease by 30 kN/m²

e) Increase by 40 kN/m²

8. If the angle of internal friction decreases, then Ka:

a) Increases

b) Decreases

c) Does not necessarily increase

d) Does not necessarily decrease

e) Does not change

9. Generally the vertical displacement of the pile which is required to mobilize full point resistance is that required to mobilize full skin friction:

a) much greater than

b) much smaller than

c) equal to

d) not greater than

e) None of the above

10. The point resistance (Qp) for a single concrete pile with a length of 6 m, diameter of 0.5 m and embedded in soil with undrained cohesion of 41 kPa is equal to:

a) 369 kN

b) 41 kN

c) 246 kN

d) 72 kN

e) None of the above is correct

11. The N60 used in Briaud (1985) correlation to predict tip resistance of pile is:

a) The N60 value at the level of pile tip

b) the average of the N60 values from the NGL to the level of pile tip

c) the average of the values of 10D above and 4D below the pile tip

d) the average of the values of 10 to 25% of D above and below pile tip

e) None of the above is correct

12. The principle of critical embedded depth of piles exists for the case of:

a) Pile embedded in clay

b) piles embedded in sand

c) piles embedded in undrained soil

d) piles embedded in collapsible soil

e) Piles embedded in loess

13. Ko of a normally consolidated soil is typically in the range of:

a) 0.4 – 0.6

b) 1.0-1.2

c) 1.4-2.0

d) 2.0-2.5

e) All of the above

14. The frictional resistance of pile in sand varies with depth till a critical depth and then it will be constant downward to the end of the pile. This depth equals:

a) (8-10) D

b) (10-12) D

c) (15-20) D

d) (22-25) D

e) (25-30) D

15. Gravity retaining wall type is suitable for retaining backfill upto:

a) 3-5 m

b) 5-8 m

c) 4 - 7 m

d) Any height

e) None of the above

16. What factor influences the nature of the variation of unit skin friction (f) in the field?

a) Pile diameter

b) Depth

c) Soil type

d) Installation method

e) Pile material

17. What condition warrants the use of point bearing piles?

a) When there is no stiff stratum within reasonable depth

b) When relying on shaft resistance becomes necessary

c) When the soil near the ground surface is weak

d) When the load is evenly distributed along the pile length

e) When using compaction piles becomes cost-effective

18. How does increasing the number of piles in a group typically affect elastic settlement?

a) It decreases settlement due to increased load-sharing among piles

b) It has no effect on settlement

c) It may increase settlement due to soil compression between piles

d) It increases settlement due to reduced pile stiffness

e) It reduces settlement due to increased lateral support between piles

19. The value of σ'h at failure for rest, active, and passive state is as follows:

a) (σ'h)rest < (σ'h)Passive ≤ (σ'h)Active

b) (σ'h) Active < (σ'h) rest < (σ'h) Passive

c) (σ'h)Passive < (σ'h) rest < (σ'h)Active

d) (σ'h)Passive < (σ'h) Active < (σ'h) rest

e) (σ'h) Active > (σ'h) rest > (σ'h) Passive

20. A wall retained a soil of depth of 5 m. if the water table located at depth 2 m below ground surface. for undrained condition, the pore water pressure exerted on the wall is: (Φ=30°, γw = 10 kN/m³, γt = 17kN/m³, and γSat = 19 kN/m³):

a) zero

b) 45 kN/m

c) 30 kN/m

d) 90 kN/m

e) 27 kN/m

21. The most common type of retaining walls is:

a) Gravity retaining walls

b) Semigravity retaining walls

c) Cantilever retaining walls

d) Counterfort retaining walls

e) All of above

22. What is the main contributor to downward drag forces known as negative skin friction?

a) Cyclic loads during testing

b) Impact from vibratory hammers

c) Failure in the pile splices

d) Friction reduction between soil and pile

e) Soil consolidation

23. What alteration to the retaining wall structure can be made in addition to elongating the base slab to improve the factors of safety against sliding?

a) Create a deeper foundation for the wall

b) Use a key to the base slab

c) Dig a trench along the footing for added stability

d) Connect the wall to an adjacent more stable structure

e) Add a counterfort to the retaining wall

24. Which type of piles are referred to as nondisplacement piles?

a) H-piles and open-ended pipe piles

b) Piles advanced by partial augering

c) Steel sheet piles

d) Prestressed concrete piles

e) Cast-in-place (or bored) piles

25. What is the nature of variation of the lateral pressure with the magnitude of wall movement?

a) The wall movement required to mobilize the passive state is less than that required to mobilize the active state

b) The wall movement required to mobilize the at-rest state is less than that required to mobilize the active state

c) The wall movement required to mobilize the at-rest state is less than that required to mobilize the passive state

d) The wall movement required to mobilize the at-rest state is equal to that required to mobilize the active state

e) The wall movement required to mobilize the active state is less than that required to mobilize the passive state

26. What are compaction piles?

a) Piles driven into cohesive soil to achieve proper compaction of soil deep below the ground surface

b) Piles driven into cohesive soil to achieve proper compaction of soil close to the ground surface

c) Piles driven into granular soil to achieve proper compaction of soil close to the ground surface

d) Piles driven into granular and cohesive soil to achieve proper compaction of soil close to the ground surface

e) Piles driven into granular soil to achieve proper compaction of soil deep below the ground surface

27. The downward drag acting on a pile due to the movement of the surrounding is called:

a) Skin friction

b) Negative skin friction

c) Frictional force

d) None of the mentioned

e) Point bearing

28. The wall movement required to mobilize the active state is __ than that required to mobilize the passive state:

a) slightly greater

b) greater

c) less

d) alike

e) very larger

29. One of the elastic settlement in pile foundation according to Vesic (1977), is the settlement of the pile due to:

a) the ultimate bearing capacity

b) the allowable total bearing capacity

c) the ultimate load carrying capacity

d) the working load at the pile point

e) the permissible bearing capacity

30. The wall may tilt away from the soil that is retained. With sufficient wall tilt, a triangular soil wedge behind the wall will fail. The lateral pressure for this condition is referred to as:

a) active earth pressure

b) at rest coefficient

c) passive earth pressure

d) both of a and b

e) none of these

31. Pile foundations are structural member where the depth is __ than the width:

a) greater

b) larger

c) considerably longer

d) equal or less

e) approximately less

32. A gravity retaining wall is shown in Figure using Rankine active pressure. Calculate The factor of safety against sliding. Use ΣV= 360.77 kN/m, PP= 186.59 kN/m, Ph= 126.65 kN/m:

a) F.S = 2.84

b) F.S = 2.48

c) F.S = 2.34

d) F.S = 2.2

e) F.S = 2

33. Cast-in-place piles (or bored piles) literally cause no displacement and are known as:

a) Displacement piles

b) nondisplacement piles

c) compaction piles

d) Both a and b

e) not above

34. The point bearing capacity, qp, of a pile in sand generally increases with the depth of embedment in the bearing stratum and reaches a maximum value at an embedment ratio of:

a) Lb/D > (Lb/D)cr

b) Lb/D = (Lb/D)cr

c) Lb/D < (Lb/D)cr

d) Both a and b

e) Both b and c

35. When hard driving conditions are expected, such as driving through dense gravel, shale, or soft rock. Steel piles can be fitted with:

a) Epoxy coating

b) driving points or shoes

c) All above

d) Paint coating

e) Not above

36. The length of compaction piles depend on:

a) relative density of the soil before and after the compaction

b) the required depth of compaction

c) both a and b

d) the angle friction

e) Not above

37. Consider a Pipe pile having an outside diameter of 457 mm. The embedded length of the pile in layered saturated clay is 20 m. The groundwater table is located at a depth of 3 m from the ground surface. Estimate Qp by using Meyerhof's method (Cu at 10-30m = 90 kN/m²):

a) Qp ≈ 37 kN

b) Qp ≈ 60 kN

c) Qp ≈ 133 kN

d) Qp ≈ 1300 kN

e) Qp ≈ 1500 kN

38. At elastic settlement of piles the magnitude of ξ varies between:

1) 0.50 to 0.60

2) 0.50 to 0.67

3) 0.57 to 0.65

4) 0.57 to 0.67

5) 0.55 to 0.65

39. Concrete piles may be divided into two basic categories:

a) Precast and cast-in-situ piles

b) Friction and point load piles

c) Open and closed end piles

d) Cased and uncased piles

e) all of the mentioned

40. The term γDNγ* may be dropped from the equation of ultimate resistance of pile because:

a) γ is small

b) the D is small

c) the D is large

d) Nγ* is zero

e) γ is large

41. The main purpose of the shoes (Driving point) in steel piles is:

a) Extend the length of the pile

b) Enlarge the base of the pile

c) Add resistance to chemical materials

d) Protect the pile end from damage when driven in hard soils

e) Protect the pile when driven in expansive soils

42. Consider a pipe pile having an outside diameter of 457 mm. The embedded length of the pile in layered saturated clay is 20 m. The subsoil properties from 10 – 30 m is Y = 18 kN/m³ and Cu = 90 KN/m². Using Meyerhof's method, the ultimate point Qp can be approximated to:

a) 133 KN

b) 333 KN

c) 1033 KN

d) 34 KN

e) 170 KN

43. When determining the capacity of a group pile in clay, the equation ΣQu = n1 n2 (Qp + Qs) assumes:

a) The piles in the group act as a block (group)

b) The Piles in the group subjected to uplift pressure

c) The piles in the group undergo negative skin pressure

d) The piles in the group act as summation of the individual pile capacities

e) The Piles in the group subjected to moment in both directions

44. If the backfill carries a uniform surcharge q, then the lateral pressure at the depth of wall H is:

a) pa = Ka γz + Ka q

b) pa = Ka γz - Ka q

c) pa = Ka γz * Ka q

d) pa = Ka γz / Ka q

e) pa = Ka γz + q

45. In the field, for full mobilization of the point resistance (Qp), the pile tip must go through a displacement of:

a) 8 to 20% of the pile width (or diameter)

b) 10 to 25% of the pile width (or diameter)

c) 10 to 50% of the pile width (or diameter)

d) 25 to 50% of the pile width (or diameter)

e) 10 to 15% of the pile width (or diameter)

46. When the wall restrained from moving, the lateral earth pressure on the wall at any depth is called the:

a) Passive earth pressure

b) active earth pressure

c) at-rest earth pressure

d) vertical pressure

e) none of the above

47. The coefficient of earth pressure at rest is represented as expression given by Jaky:

a) K0=1-sinφ

b) K0=sinφ

c) K0=1-cosφ

d) K0=cosφ

e) K0= tanφ

48. The area of the pile group along the failure surface is equal to:

a) Perimeter × Area of cross section

b) Breadth × Length

c) Perimeter × Length

d) Perimeter / Length

e) Perimeter/area of cross section

49. The settlement of a group of friction piles can be computed on the assumption that:

a) Clay is incompressible

b) Pile below the lower level is ignored

c) Bearing resistance is zero

d) All of the mentioned

e) None of the mentioned

50. The pile load test should be performed on:

a) Working piles

b) Trial piles

c) Working and trial piles

d) Soil

e) None of the mentioned

51. In practice, the minimum center-to-center pile spacing, d, is:

a) 2.5D

b) 3D

c) 3.5D

d) 2D to 3D

e) 2.5D to 3D

52. If the wall is prevented from moving, then the lateral pressure of the earth on the wall at any depth is called:

a) the Active earth pressure

b) the Passive earth pressure

c) the at-rest earth pressure

d) All of the mentioned

e) None of the mentioned

53. Different types of piles are used in practice, depending on:

a) load to be carried

b) soil conditions

c) location of the water table

d) the installation technique

e) All mentioned above

54. A concrete pile 15.24 m long having a cross section of 406 mm x 406 mm is fully embedded in a saturated clay layer for which γ= 19.02 kN/m³, and cu =76.7 kN/m². Use the λ method to estimate the skin friction:

a) 194.86

b) 294.86

c) 394.86

d) 494.86

e) 594.86

55. The pile shaft will be fully mobilized when the relative displacement between the soil and the pile is about:

a) 5 mm

b) 10 mm

c) 5 cm

d) 10 cm

e) 5-10 mm