Foundation Engineering 2 - Exam Questions

Exam Date: 27/5/2024  |  Time: 120 minutes  |  Stage: Fourth

Note: Correct answers are highlighted in green.

1- If the required factor of safety against sliding is not achieved, then the most practical choice is:

a- Increasing the angle of soil backfill

b- Increasing surcharge

c- Increasing retaining wall dimensions

d- Using a key to the base slab

e- None of the above

2- When hard driving conditions are expected, such as driving through dense gravel, shale, or soft rock, steel piles can be fitted with:

a- Epoxy coating

b- driving points or shoes

c- All above

d- Paint coating

e- Not above

3- These piles may be divided into two broad categories: (a) cased and (b) uncased:

a- Cast-in-situ

b- Precast piles

c- Steel piles

d- Both a and b

e- not above

4- Techniques can be used for a pile load test:

a- load-controlled test

b- cyclic loading test

c- rate-of-penetration test

d- all of a,b,c

e- none of a,b,c

5- The critical surface of sliding is:

a- The failure surface along which the minimum factor of safety is obtained

b- The failure surface along which the larger factor of safety is obtained

c- The failure surface along which the maximum factor of safety is obtained

d- failure surface at a depth of about 1.5 times the width of base slab of the retaining

e- None of the above

6- Design of gravity wall aims to:

a- resist appreciable tension

b- safe against sliding

c- safe against overturning

d- all of the above

e- none of the above

7- Proportioning is:

a- some dimensions to allow recheck the section of wall for stability

b- the height of the retaining wall

c- the stem of the retaining wall

d- the base of the retaining wall

e- the depth of the retaining wall

8- The pre-stressed concrete piles as compared to pre-cast and reinforced concrete piles:

a- are lesser in weight

b- have high load carrying capacity

c- are extremely durable

d- are Highly length

e- All of the mentioned

9- Piles are structural members made of:

a- Steel

b- Concrete

c- Timber

d- Composite

e- all of above

10- Back fill with a sloping surface exerts a total active force Pa on the wall of height H and acts at:

a- H/4 above the base parallel to base

b- H/2 above the base parallel to base

c- H/3 above the base parallel to base

d- H/5 above the base parallel to base

e- H/6 above the base parallel to base

11- If the desired value of Fs (sliding) is not achieved in retaining wall:

a- the increase in width of the base slab may be used

b- the reduce in width of the base slab may be used

c- increase surcharge load

d- the key below base in not used

e- all the above

12- How do gravity retaining walls achieve stability?

a- They depend for stability on their own weight and any soil resting on the masonry

b- By relying on external braces

c- By using soil anchors

d- By incorporating steel reinforcement

e- None of the above

13- In what ways may a retaining wall fail?

a- It may overturn about its toe

b- It may slide along its base

c- It may undergo deep-seated shear failure

d- It may go through excessive settlement

e- all of them

14- The purpose of the counterforts in retaining walls is:

a- Reduce the shear

b- Reduce the weight

c- Reduce the size

d- all of the mentioned

e- none of the mentioned

15- Types of sheet pile are commonly used in construction:

a- wooden sheet piles

b- precast concrete sheet piles

c- steel sheet piles

d- Aluminum sheet piles are also marketed

e- all of above

16- What are the two phases in the design of a conventional retaining wall?

a- Determining the aesthetic value and cost effectiveness

b- Calculating the base slab and reinforcing the stem

c- Checking the structure for stability and checking components for strength

d- Evaluating landscape integration and potential for height extension

e- Checking for weight distribution and decoration locations

17- What alteration to the retaining wall structure can be made in addition to elongating the base slab to improve the factors of safety against sliding?

a- Create a deeper foundation for the wall

b- Use a key to the base slab

c- Dig a trench along the footing for added stability

d- Connect the wall to an adjacent more stable structure

e- Add a counterfort to the retaining wall

18- The factor of safety against overturning of retaining walls is most nearly:

a- 1.5

b- 4.5

c- 2.0

d- 1

e- 5

19- The shear key is provided to:

a- Avoid friction behind the wall

b- Improve appearance

c- Increase passive resistance

d- All of the above

e- None of the above

20- Design of gravity wall aims to:

a- resist appreciable tension

b- safe against sliding

c- safe against overturning

d- all are correct

e- None of the above

21- A minimum factor of safety against sliding is recommended as:

a- 2

b- 2.5

c- 1.5

d- 3

e- 5

22- The shear key is provided to:

a- Avoid friction behind the wall

b- improve appearance

c- Increase sliding resistance

d- All of the mentioned

e- None of the above

23- Which one used as deep groundwater barriers through and under dams:

a- Retaining wall

b- Diaphragm wall

c- Sheet pile wall

d- All are correct

e- None of the above

24- The displacement at which friction resistance is fully mobilized is:

a- 5-10 mm

b- 10% of the pile diameter

c- 25% of the pile diameter

d- 40 mm

e- 62 mm

25- Retaining wall that is made of reinforced concrete of a thin stem and a base slab is called:

a- Gravity retaining wall

b- Geosynthetic retaining wall

c- Semi gravity retaining wall

d- Cantilever retaining wall

e- Gabon retaining wall

26- A retaining wall with H = 7.158 m, Ph = 158.75 kN/m, Summation of MR (The resisting moment) = 1128.86 kN.m/m. The approximate factor of safety against overturning is:

a- 3

b- 2

c- 2.5

d- 4

e- 1.5

27- To reduce the shear and the bending moments, a thin vertical concrete slabs can be used in the retaining walls, these slabs are called:

a- Stems

b- Heels

c- Toes

d- base

e- counterforts

28- To design retaining walls properly, an engineer must know the following basic geotechnical parameters:

a- Φ, γ and C for retained soil

b- Φ, γ, and C for retained soil and soil below the base

c- Φ, and C for retained soil

d- Φ, and C for the soil below the base

e- Φ, and C for retained soil and soil below the base

29- In Coulomb theory for active lateral pressure: the line of action of the resultant force (P acts in inclined at a distance above the base of the wall):

a- H

b- H/2

c- H/3

d- H/6

e- Undefined

30- The soil used for backfill behind the sheet-pile wall is usually:

a- Sand or gravel

b- Silt or clay

c- Sand or silt

d- Clay or sand

e- Gravel or silt

31- On designing retaining walls, it is necessary to take care of exerted by soil mass:

a- Erosion

b- Lateral pressure

c- Surcharge

d- Effective pressure

e- Total pressure

32- Which of the following piles is used to compact loose granular soil?

a- Friction piles

b- End bearing piles

c- Compaction piles

d- Tension piles

e- Sheet piles

33- Assuming that σall = 172.5 MN/m² and maximum moment = 103.59 kN.m/m, the minimum size of sheet-pile section necessary is:

a- 6 x10^-4 m3/m

b- 2 x10^-3 m3/m

c- 8 x10^-2 m3/m

d- 9 x10^-1 m3/m

e- No one of above

34- The stability of cantilever sheet pile depends mainly on:

a- Embedment depth below dredge line

b- Weight of sheet pile

c- Height of sheet pile

d- Shear strength parameters of backfill soil

e- None of the above

35- In pile cyclic load test:

a- An incremental load is repeatedly applied and removed

b- The load on the pile is continuously increased to maintain a constant rate of penetration

c- Requires the application of step loads on the piles and the measurement of settlement

d- are applied in increments as specified by the relevant standards, with sufficient

e- Pile cyclic load test is not standard test

36- The pile hammer is selected based on:

a- the pile weight and the soil characteristics

b- the soil characteristics and parameters

c- the pile weight and width

d- the pile weight and length

e- the pile weight and its dimension

37- Which type of retaining wall depends on its weight and any soil resting on the masonry for stability?

a- Gravity wall

b- Cantilever wall

c- Counterfort wall

d- Anchored wall

e- None of the above

38- For deep shear failure analysis of soil underneath a retaining wall, where is the critical cylindrical failure surface most likely located?

a- Immediately below the base slab

b- At a depth of 0.5 times the base slab width

c- At a depth of 1.5 times the base slab width

d- Below the critical backfill slope angle

e- At the intersection of the backfill and foundation soil

39- In designing a retaining wall, Rankine's active earth pressure theory for stability checks involves drawing a vertical line through point located at:

a- Top of the stem

b- Edge of the toe

c- Center of the base slab

d- Bottom of the base slab

e- Edge of the heel of the base slab

40- The necessary amount of outward displacement of the wall is about for cohesive soil backfills, where H is the height of the wall:

a- 0.001H to 0.002H

b- 0.001H to 0.003H

c- 0.001H to 0.004H

d- 0.002H to 0.004H

e- 0.0003H to 0.0004H

41- The Rankine active earth pressure calculations were based on the assumption that the wall is:

a- smooth to rough

b- rough

c- friction

d- frictionless

e- smooth to very rough

42- Counterfort retaining walls are similar to cantilever walls. At regular intervals, however, they have thin vertical concrete slabs known as counterforts that:

a- tie wall and the soil together

b- tie the wall and the base slab together

c- tie the base slab and the soil together

d- All of above

e- none of the above

43- The counterfort retaining wall slabs may be spaced at center-to-center distances of:

a- 0.2H

b- 0.8H

c- 0.2H to 0.5H

d- 0.3H to 0.7H

e- 0.5 to 0.7H

44- Cantilever sheet-pile walls are usually recommended for walls of moderate height or less, measured above the dredge line:

a- 9 m

b- 6 m

c- 2 m

d- 8 m

e- 12 m

45- In loose sand at similar depths, what type of pile is likely to have a higher unit skin friction?

a- Low-displacement pile

b- High-displacement pile

c- Bored pile

d- Jetted pile

e- Vibro pile

46- What is the purpose of constructing a pile cap over group piles?

a- To increase the load-bearing capacity of individual piles

b- To reduce the spacing between piles

c- To transmit the structural load to the soil

d- To minimize the effect of negative skin friction

e- To measure the settlement of each pile

47- The purpose of used the counterforts in Counterfort retaining walls is to reduce:

a- The shear and the bending moments

b- width

c- height

d- width and height

e- cost

48- Pick up the incorrect statement from the following:

a- In the stem of a retaining wall, reinforcement is provided near the earth side

b- In the toe slab of a retaining wall, reinforcement is provided at the bottom of the slab

c- In the heel slab of a retaining wall, reinforcement is provided at the top of the slab

d- All of these

e- None of these

49- All are the component of retaining wall except?

a- Toe Slab

b- Heel Slab

c- Surcharge

d- Stem

e- Base slab

50- Design of gravity wall aims to:

a- Safe against excessive settlement

b- Safe against sliding

c- Safe against overturning

d- All are correct

e- All are not correct

51- For the design of retaining walls, the minimum factor of safety against sliding is taken as:

a- 1

b- 1.5

c- 2

d- 2.5

e- 2 to 3

52- Types of failure in retaining walls are:

a- Overturning

b- Sliding

c- Bearing capacity failure

d- Deep-seated shear failure

e- All of the above

53- For the design of retaining walls, the minimum factor of safety against Bearing Capacity is:

a- 1

b- 2

c- 3

d- 4

e- 2 to 3

54- As compared to the cantilever sheet pile, the required depth of anchored sheet pile below the dredge line is:

a- more

b- less

c- same

d- all are correct

e- all are wrong

55- Precast concrete piles driven in granular soils to achieve:

a- Proper compaction

b- Proper tension

c- Compaction and tension

d- All the above points

e- None of the above points