Ministry of Higher Education and Scientific Research

Second Round Exam - Fourth Stage - Foundations 2

Date: 30/6/2024     Time: 120 minutes

Note: Correct answers are highlighted in green

1- The dimensions of the proportioning retaining walls depends on:

a- Soil properties

b- Type of retaining wall

c- Theory used for lateral pressure calculations

d- Height of backfill

e- None of the above

2- The minimum factor of safety for a retaining wall to avoid overturning is:

a- 1.0

b- 1.5

c- 2.0

d- 2.5

e- 3.0

3- H piles is considered a:

a- No displacement piles

b- highly displacement pile

c- low displacement pile

d- cast in situ pile

e- none of the options is correct

4- The overturning force of a retaining wall is:

a- Horizontal active force

b- Vertical active force

c- Passive force

d- Weight of the retaining wall

e- Weight of soil

5- The soil backfill directly above the base of a retaining wall causes:

a- Overturning

b- Sliding

c- Bearing capacity failure

d- All of the above

e- None of the above

6- Sheet piles are used to build a continuous wall to:

a- Highways

b- Waterfront structures

c- Towers

d- Multistory-buildings

e- Trusses

7- In cantilever sheet piles, a special care should be taken to:

a- Soil properties

b- Height of sheet pile

c- Fluctuation of water table

d- Sheet pile material

e- None of the above

8- Piles are driven into granular soil to achieve proper compaction of soil. These piles are called:

a- compaction piles

b- batter piles

c- Both a and b

d- Belled piles

e- not above

9- The Rankine active earth pressure coefficient for a soil with Ø = 0 is:

a- zero

b- 1

c- -1

d- 2

e- -2

10- Sheet-pile walls:

a- need dewatering of the site

b- do not need dewatering of the site

c- do not used for waterfront structures

d- all of the above

e- none of the above

11- The critical surface of sliding is:

a- The failure surface along which the minimum factor of safety is obtained

b- The failure surface along which the larger factor of safety is obtained

c- The failure surface along which the maximum factor of safety is obtained

d- Failure surface at a depth of about 1.5 times the width of base slab of the retaining

e- None of the above

12- A retaining wall may fail in:

a- overturn about its toe

b- slide along its base

c- fail due to the loss of bearing capacity of the soil

d- fail due to excessive settlement

e- All of above

13- A minimum factor of safety against sliding is recommended as:

a- 2

b- 2.5

c- 1.5

d- 3

e- 4

14- The pre-stressed concrete piles as compared to pre-cast and reinforced concrete piles:

a- are lesser in weight

b- have high load carrying capacity

c- are extremely durable

d- are Highly length

e- All of the mentioned

15- Design of gravity wall aims to:

a- resist excessive settlement

b- safe against sliding

c- safe against overturning

d- safe bearing capacity

e- All of the mentioned

16- On designing retaining walls it is necessary to take care of ____ exerted by soil mass:

a- Erosion

b- lateral pressure

c- Surcharge

d- vertical stress

e- point load

17- What is the primary function of a retaining wall?

a- To provides lateral support for a vertical or near-vertical slope of soil

b- To provide horizontal slope

c- To prevent erosion of a slope

d- To enhance the aesthetics of a slope

e- None of the above

18- Wooden sheet piles are used only for:

a- temporary, light structures that are above the water table

b- permanent, light structures that are above the water table

c- temporary, heavy structures that are above the water table

d- temporary, light structures that are under the water table

e- none of the mentioned

19- A retaining wall 5m high; unit weight of soil =17.6 kN/m³; Ka =0.2444. Calculate the Coulomb's active force per unit length of the wall.

a- 53.77 kN/m

b- 67.0 kN/m

c- 30.0 kN/m

d- 101.0 kN/m

e- No one of above

20- What is the minimum diameter is specified for Class C timber piles used permanently under the water table?

a- 150 mm

b- 330 mm

c- 200 mm

d- 356 mm

e- 305 mm

21- When the soil conditions near the surface are poor, ............ must be used:

a- Shallow foundations

b- Deep foundation

c- Both a and b

d- earth structure

e- Not above

22- The relation between Kp and Ka is ____:

a- Kp=Ka

b- Kp=1+Ka

c- Kp=1-Ka

d- Kp=1/Ka

e- Kp=1/(1-Ka)

23- What are the common shapes of sheet-pile sections?

a- Circular and triangular

b- Z, deep arch, low arch, or straight web

c- Square and rectangular

d- Elliptical and hexagonal

e- deep arch and Z

24- Cantilever retaining walls is suitable for retaining backfill about __:

a- 8m

b- 12m

c- 2m

d- 4m

e- Any height

25- Compared to dry backfill, submerged backfill will exert ---------:

a- same earth pressure

b- less earth pressure

c- more earth pressure

d- difficult to tell

e- None of the above

26- The shear key is provided to ----------:

a- Avoid friction behind the wall

b- Improve appearance

c- Increase passive resistance

d- All of the above

e- None of the above

27- Cast-in-place piles (or bored piles) is considered to be:

a- non-displacement piles

b- high displacement piles

c- low displacement piles

d- No one of above

e- All of above

28- The wall friction of retaining wall:

a- Decreases active earth pressure but increases passive earth pressure

b- Decreases passive earth pressure but increases active earth pressure

c- decrease both active and passive earth pressure

d- increases both active and passive earth pressure

e- None of the above

29- When determining the capacity of a group pile in clay, the equation ΣQu=n1 n2 (Qp+Qs) assumes:

a- The piles in the group act as a block (group)

b- The Piles in the group subjected to uplift pressure

c- The piles in the group undergo negative skin pressure

d- The piles in the group act individually and the total capacity is the summation of the individual pile capacities

e- The Piles in the group subjected to moment in both directions

30- The best Sheet-pile walls for retaining a soil cut deeper than 6 m is:

a- Anchored sheet pile

b- Cantilever sheet pile

c- Both A and B

d- Neither A nor B

e- Secant pile

31- When the soil is at rest condition, the wall tend to ------------:

a- Move towards the supported soil

b- Move away from the supported soil

c- Still without movement

d- Have excessive settlement

e- Have shear failure

32- The group efficiency of driven pile group in sand at a close spacing may be:

a- Equal to 1

b- Well below 1

c- Greater than 1

d- below 0.5

e- None of above

33- Vibratory pile is effective with ................:

a- Precast piles

b- Bored piles

c- Timber piles

d- Steel piles

e- All of the mentioned

34- The Coefficient of passive pressure (Kp) is------------------if the Coefficient of active pressure (Ka) is 0.25:

a- 0.33

b- 4.0

c- 0.25

d- 0.5

e- 1

35- A cantilever sheet pile derives its stability from:

a- lateral resistance of soil

b- self-weight

c- the dead man

d- the anchor rods

e- surcharge

36- How is the base-level bending moment of a cantilever retaining wall expressed as a function of its height H?

a- H

b- H/2

c- H/3

d- H/6

e- Undefined

37- Which one of the following is the correct assumption of Rankine's theory?

a- The soil mass is infinite

b- The soil mass is non homogeneous

c- The soil mass is cohesive

d- The ground surface is a plane which may be horizontal or inclined

e- All of the mentioned

38- The active earth pressure of a soil is proportional to (where Ø is the angle of friction of the soil):

a- tan (45° - Ø)

b- tan (45° + Ø)

c- tan² (45° - Ø/2)

d- tan² (45° + Ø/2)

e- None of the above

39- The common types of retaining wall may be classified as follows:

a- Gravity retaining walls

b- Semi-gravity retaining walls

c- Cantilever retaining walls

d- Counterfort retaining walls

e- All the mentioned

40- The movement of soil under the action of load can be prevented by confining the ground by use of:

a- Steel pile

b- Concrete pile

c- Timber pile

d- Sheet pile

e- None of the above

41- The intensity of the active pressure for sheet piles at a depth Z = L₁ is:

a- 0.5 γ L₁ Ka

b- γ L₁ Ka

c- 2 γ L₁ Ka

d- γ L₁ Kp

e- 0.5 γ L₁ Kp

42- Obtaining the maximum moment (Mmax) per unit length of the wall at shear equal to:

a- Maximum Force

b- Minimum Force

c- Zero

d- The passive earth pressure

e- The active earth pressure

43- Related to Stability of RW, Which of the following does NOT represent a potential failure mode for a retaining wall?

a- Bearing capacity failure of the foundation soil

b- Wall cracking due to thermal expansion

c- Excessive settlement due to weak soil layer

d- Shear failure within the foundation soil adjacent to the wall

e- Sliding along the base due to insufficient friction

44- If the desired factor of safety against sliding is not met, which strategy is NOT a suitable solution?

a- Increase base slab width

b- Use a key to the base slab

c- Use a deadman anchor

d- Reduce the backfill angle

e- All of the above

45- The maximum point resistance will not be mobilized until the tip of the pile has moved about:

a- (5 to 10%) D

b- (15 to 25%) D

c- (10 to 25%) D

d- (7 to 25%) D

e- (10 to 20%) D

46- In contrast to the construction of other types of retaining walls, the building of sheet-pile walls does not usually require ---------------- of the site:

a- soaking

b- defreezing

c- polluting

d- sinking

e- dewatering

47- What is the equation for the ultimate load-carrying capacity of a pile?

a- Qu = Qp - Qs

b- Qp = Qu + Qs

c- Qs = Qu + Qp

d- Qu = Qp + Qs

e- none of them

48- The factor that is responsible for inclination of resultant pressure to the retaining wall is:

a- Frictional force

b- Surcharge

c- Earth pressure

d- Weight of the wall

e- All of the mentioned

49- When there is no stiff stratum within a reasonable depth:

a- Qu= Qp + Qs

b- Qu= Qp

c- Qu= Qs

d- all of the mentioned

e- none of the mentioned

50- A sheet pile subjected to a concentrated horizontal load at its top is known as:

a- Cantilever sheet pile

b- Diaphragm sheet pile

c- Free cantilever sheet pile

d- Anchored sheet pile walls

e- All of the above

51- The point resistance (Qp) of a driven square pile of width 0.4 m in saturated clay that has a cohesion of 100 kN/m² equal to ---------:

a- 90 ton

b- 80 ton

c- 144 ton

d- 900 kN

e- 1224 kN

52- The pressure distribution diagram for earth pressure at rest is:

a- rectangular

b- circular

c- constant

d- triangular

e- square

53- Which statement is correct?

a- Ko>KP>Ka

b- Ko>KP>Ka

c- KP>Ko>Ka

d- Ka>KP>Ko

e- No one of above

54- The coefficient of earth pressure at rest is represented as ________ expression given by Jaky:

a- ko=1-sinφ

b- ko=sinφ

c- ko=1-cosφ

d- ko=cosφ

e- ko= tanφ

55- What structure behavior implies the action of passive earth pressure?

a- The wall being under heavy loads

b- The wall inclining away from the soil

c- The wall contracting due to cold

d- The wall being pushed into the soil

e- The wall extending vertically