Ministry of Higher Education and Scientific Research

Second Round Exam - Fourth Stage - Foundations 2

Date: 10/9/2025     Time: 120 minutes

Note: Correct answers are highlighted in green

1- Reinforcement is included in concrete piles primarily to resist:

a- Shear stress during driving

b- Axial compression during service

c- Bending moment during pickup and transportation

d- Uplift forces from foundation settlements

e- Not above

2- The factor of safety in piles generally used ranges from:

a- 1 to 2

b- 2 to 3

c- 3 to 4

d- 5 to 6

e- No one of above

3- In general, the settlement of pile group in saturated clay layer is .......... sandy layer:

a- equal

b- more than

c- less than

d- less or equal

e- None of the above

4- Piles are recommended in the following situations:

a- In weak ground conditions

b- For carrying lateral loads

c- In expansive or collapsible soil

d- For resisting uplift

e- All of above

5- For the equation f=kσ' tanδ', K is ............. for great depth:

a- Rankine active earth pressure coefficient

b- Rankine passive earth pressure coefficient

c- At-rest pressure coefficient

d- all of the mentioned

e- none of the mentioned

6- How do gravity retaining walls achieve stability?

a- They depend for stability on their own weight and any soil resting on the masonry

b- By relying on external braces

c- By using soil anchors

d- By incorporating steel reinforcement

e- None of the above

7- The minimum tip diameter for Class B timber piles should not be less than:

a- 150 mm

b- 250 mm

c- 305 mm

d- 330 mm

e- 356 mm

8- The actual depth of penetration is ............................of the theoretical depth for cantilever sheet piling penetrating clay:

a- increased by 20% to 30%

b- increased by 40% to 60%

c- the same

d- reduced by 10%

e- increased by 10%

9- Which of the following can be done to improve the factor of safety against sliding if the desired value is not met?

a- Increase the width of the base slab (heel of the footing)

b- Decrease the thickness of the base slab

c- Apply a tensile force at the top of the retaining wall

d- Shorten the cantilever arm of the retaining wall

e- Decrease the weight of the retaining wall

10- The effect of cohesion on a soil is to ____:

a- reduce both active and passive earth pressure intensities

b- increase both active and passive earth pressure intensities

c- reduce active earth pressure intensity but to increase passive earth pressure intensity

d- increase active earth pressure intensity but to reduce passive earth pressure intensity

e- reduce active and passive earth pressure intensity

11- A 500 mm diameter and 20 m long concrete pile is driven into a sand where γ=18.5 kN/m³ and Ø=32°. The point loading of pile is ____:

a- 4580 kN

b- 496 kN

c- 2870 kN

d- 1970 kN

e- 3560 kN

12- A retaining wall of height H with a smooth vertical back face supports a backfill inclined at an angle β with the horizontal. The backfill consists of cohesionless soil having an angle of internal friction φ. If the active lateral thrust acting on the wall is Pa, which one of the following statements is TRUE?

a- Pa acts at a height H/2 from the base of the wall and at an angle β with the horizontal

b- Pa acts at a height H/2 from the base of the wall and at an angle φ with the horizontal

c- Pa acts at a height H/3 from the base of the wall and at an angle β with the horizontal

d- Pa acts at a height H/3 from the base of the wall and at an angle φ with the horizontal

e- Pa acts at a height 2H/3 from the base of the wall and at an angle β with the horizontal

13- When structural load is heavy and/or soil having low bearing capacity for considerable depth, the suitable foundation for this situation is:

a- Shallow footings

b- Piles

c- Rafts

d- Wells or Piers

e- Combined footing

14- While driving a large number of piles in loose sand:

a- It is advantageous to follow a sequence of pile driving such that the inner piles are driven first and then proceed outwards

b- It is advantageous to follow a sequence of pile driving such that the piles near the periphery are driven first and inner piles are driven later

c- It is advantages to follow a sequence of pile driving such that alternatively inner and outer piles are driven

d- Driving of piles can be done in any random order

e- None of the above

15- The elastic compression ΔL of a pile can be calculated by the expression based on:

a- Terzaghi's theory

b- Hooke's law

c- Meyerhof's theory

d- Hiley's formula

e- None of the above

16- At similar depth, the unit frictional resistance, f, is - Variation of the unit frictional resistance for different piles:

a- Higher in low displacement piles

b- Same in all pile types

c- Higher for high displacement piles

d- Higher for bored piles

e- Higher in H piles

17- Nowadays, fiber reinforced polymer (FRP) composite piles are widely used for:

a- waterfront structures

b- Basements

c- Residential Building

d- Seismic Condition

e- None of the mention

18- Connected or semi-connected ................are often used to build continuous walls for waterfront structures that range from small waterfront pleasure boat launching facilities to large dock facilities:

a- Steel piles

b- Concrete piles

c- Timber piles

d- Sheet piles

e- None of the mentioned

19- In prestressed precast piles, During casting of the piles, the cables are ..........pretensioned to about, and concrete is poured around them:

a- 900 to 1300 MN/m²

b- 600 to 800 MN/m²

c- 500 to 700 MN/m²

d- 1500 to 2000 MN/m²

e- None of the mention

20- Based on Meyerhof SPT formula, the ratio of frictional resistance of low displacement pile to that of high displacement pile driven in the same soil is:

a- One fourth

b- One third

c- Two third

d- One fifth

e- One half

21- The precast concrete piles are generally used for a maximum design load of about:

a- 50 tones

b- 60 tones

c- 75 tones

d- 80 tones

e- 100 tones

22- The piles that are used for protecting structures from ships and floating object is:

a- Anchor piles

b- Fender piles

c- Compaction piles

d- Batter piles

e- Sheet piles

23- The minimum factor of safety against sliding for retaining wall would be:

a- 1.5

b- 2

c- 2.5

d- 3

e- 3.5

24- The most economical type of retaining wall used to retain a soil of height of 10 m is:

a- Gravity wall

b- Cantilever wall

c- Counterfort wall

d- Semi-gravity wall

e- Cantilever wall or Counterfort wall

25- When the piles are enlarged to provide anchorage against uplift. Such piles are known as:

a- belled or underreamed piles

b- batter piles

c- Both a and b

d- All above

e- not above

26- These piles may be divided into two broad categories: (a) cased and (b) uncased:

a- Cast-in-situ

b- Precast piles

c- Steel piles

d- Both a and b

e- not above

27- In friction piles, it is assumed that the entire load is transferred through the pile shaft in the form of friction or adhesion, with Qp is:

a- Qp > 0

b- Qp = 50% Qu

c- Qp = 0

d- Both a and b

e- Both b and c

28- Related to Stability of RW, which of the following DOES NOT represent a potential failure mode for a retaining wall?

a- Bearing capacity failure of the foundation soil

b- Wall cracking due to thermal expansion

c- Excessive settlement due to weak soil layer

d- Shear failure within the foundation soil adjacent to the wall

e- Sliding along the base due to insufficient friction

29- In designing a retaining wall, Rankine's active earth pressure theory for stability checks involves drawing a vertical line through point located at...................:

a- Top of the stem

b- Edge of the toe

c- Center of the base slab

d- Bottom of the base slab

e- Edge of the heel of the base slab

30- Deep shear failure in retaining walls can occur along a cylindrical surface as a result of the existence of a weak layer of soil underneath the wall within a depth of about ---- times the width of the base slab of the retaining wall:

a- 1.5

b- 2.0

c- 2.5

d- 3.0

e- 4.0

31- What method do many structural engineers often utilize to obtain group efficiency for friction piles, especially in sand?

a- Advanced computational simulations

b- Experimental field tests

c- Simplified analysis

d- Full-scale load testing

e- Finite element analysis

32- The purpose of used the counterforts in Counterfort retaining walls is to reduce:

a- The shear and the bending moments

b- width

c- height

d- width and height

e- cost

33- The efficiency of pile group depends upon:

a- Characteristic of pile

b- Spacing of pile

c- both of the above

d- Bearing capacity of soil

e- length of the pile

34- A uniform load of 50 kN/m² on a soil with unit weight 20 kN/m³ carried by a retaining wall is considered as an equivalent to an extra height of soil equal to:

a- 1.5 m

b- 2.0 m

c- 2.25 m

d- 2.5 m

e- 2.75 m

35- During the active state of plastic equilibrium, the retaining wall moves:

a- towards the fill

b- away from the fill

c- does not change its position

d- remains in equilibrium

e- down ward

36- The coefficient of earth pressure when the soil is at equilibrium:

a- σv /σh

b- σh /σv

c- σv × σh

d- σ1 / σ3

e- σh+σv

37- In point load piles:

a- the entire load is transferred through the pile shaft

b- the entire load is point resistance

c- the entire load is equally of (a) and (b)

d- neither (a) nor (b)

e- none of the above

38- Ultimate bearing capacity of pile group increase when:

a- Increasing pile number

b- reduce pile number

c- increase soil cohesion

d- increase soil angle of internal friction

e- reducing soil cohesion

39- When selecting the type of retaining wall, the following factor is most important (select the INCORRECT answer):

a- Cost and aesthetic preference

b- The type of soil and water load behind the wall

c- The wall's color and texture

d- The wall's height and width

e- The weather conditions at the site

40- H-section piles are usually preferred on I-section beam:

a- Because their web and flange length are equal

b- Because their web thicknesses are longer than the flange thicknesses

c- Because their web and flange thicknesses are equal

d- Because their web thicknesses are smaller than the flange thicknesses

e- none of the above

41- What is the definition of a retaining wall?

a- It is wall that provides lateral support for a vertical slop of soil

b- It is wall that provides lateral support for a horizontal slope of soil

c- It is wall that provides lateral support for a vertical or near vertical slope of soil

d- Both a, b

e- None of the above is correct

42- What is the range of the factor ξ used in the elastic settlement equation for piles?

a- 0.1 to 0.3

b- 0.3 to 0.5

c- 0.5 to 0.67

d- 0.67 to 0.9

e- None of the above is correct

43- The purpose of using a key to the base slab in retaining walls is:

a- Prevent overturning

b- Prevent sliding

c- Prevent bearing capacity failure

d- Prevent deep-seated shear failure

e- None of the above is correct

44- The load test procedure requires the application of step loads on the piles and the measurement of settlement and is called:

a- The constant-rate-of-penetration test

b- cyclic loading

c- The load-controlled test

d- Rebound loading

e- No one of these

45- The downward drag force exerted on a pile by the soil surrounding it is known as:

a- Negative Skin Friction

b- Friction force

c- Bearing force

d- Tension force

e- Compression force

46- The wooden sheet piles usually used for:

a- temporary, heavy structures below the water table

b- permanent structures above the water table

c- temporary, light structures below the water table

d- temporary, light structures above the water table

e- permanent structures below the water table

47- The typical thicknesses of steel sheet piles in the United States are:

a- 15 to 18 mm

b- 20 to 25 mm

c- 30 to 35 mm

d- 5 to 8 mm

e- 10 to 13 mm

48- The assumptions made when using the Rankine earth pressure theory for stability checks of a cantilever wall are:

a- Coulomb active condition along a vertical plane through the heel

b- No assumptions are made; it is based on empirical data

c- Rankine active condition along a vertical plane through the heel

d- Rankine passive condition along a horizontal plane through the base

e- Coulomb passive condition along a horizontal plane through the base

49- What happens at the point of rotation in a cantilever sheet pile wall?

a- Active and passive earth pressures are equal

b- Hydrostatic pressure is maximized

c- The moment is at its peak

d- The wall moves upwards because it undergoes a moment reversal

e- The sheet pile experiences zero lateral pressure

50- What distinguishes passive earth pressure from active earth pressure?

a- The direction of wall movement

b- The type of soil failure

c- The value of the earth pressure coefficient

d- All of the above

e- None of the above

51- The downward drag acting on a pile due to the movement of the surrounding soil is called:

a- Skin friction force

b- Negative skin friction force

c- Frictional force

d- Point bearing force

e- Tension force

52- What type of pile is designed primarily to transfer load through skin friction?

a- End-bearing piles

b- Friction piles

c- Composite piles

d- Timber piles

e- Pile caps

53- Steel, and precast concrete piles are generally:

a- Bored piles

b- End Bearing piles

c- Cast-in-situ piles

d- Driven piles

e- All of above

54- What is the at-rest earth pressure?

a- The lateral earth pressure on a retaining wall when the wall is at failure

b- The lateral earth pressure on a retaining wall when the wall is pushed into the soil

c- The lateral earth pressure on a retaining wall when the wall is restrained from moving

d- The lateral earth pressure on a retaining wall when the wall tilts away from the soil

e- The lateral earth pressure on a retaining wall when the wall is at rest

55- The coefficient of active earth pressure is ______ than the coefficient of passive pressure:

a- less

b- greater

c- equal to

d- high

e- more